Cell2Jack Function Description

LED (small)

No light 		: BT is not connected

Red		: BT is connected

Red Slow Flash	: BT is discoverable

LED (big)

Bright Blue		: Power ON and ON-HOOK

Dark Blue		: Power On and OFF-HOOK

Analog Phone DTMF Commands:

#*#  or 0000		: Enter pairing mode

*    or 1111		: Enable Siri / Google Talk (Rotary Feature)

2222XY		: Send XY commands

Option commands ( Enter Keys from your home phone) (red is default)

89#	: Ring back testing

2#	: Normal LED display

3#	: Debugging LED display mode

10#	: General Dialing Plan with 10 sec timeout

11#	: General Dialing Plan without timeout

12#   : # only Dialing Plan

14# 	: Enable Call Screening

15#	: Disable Call Screening

18# 	: Enable Rotary Feature

19#	: Disable Rotary Feature

20#	: Regular Ringing strength

21#	: Strong Ringing strength

24# ~ 28# : Speaker volume level: 0, 1, 2, 3, 4

30# ~ 33# : Hook detection 30, 50, 70, 100 ms

35#	: Enable DTMF Tone Command

36#	: Disable DTMF Tone Command

37#	: DTMF threshold Low

38#	: DTMF threshold Normal

39#	: DTMF threshold High

41#	: No dialtone after call disconnect

42#	: Play dialtone after call disconnect

43#	: Not reverse Tip/Ring after call disconnect

44#	: Reverse Tip/Ring after call disconnect

50#	: Disable CID

51#	: Enable CID

60#	: Disable (Media Audio) A2DP streaming

61#	: Enable (Media Audio) A2DP streaming

70# ~ 74#	: Off-Hook voltage: High, Medium, Low, Strong, Strongest

82#83#	: Clear Paired device and Reset parameters

80#	: Disable AEC

81#	: Enable AEC

90# ~ 94# : MIC volume level: 0, 1, 2, 3, 4

99#	: Reboot

Option Features. (Setting with PC/MAC Application)

Echo Cancel Depth Control:  Select Echo Depth.

Rotary Feature: 1111 Enable Siri or Google Voice. 5 seconds to dial independent of calling number length.

DTMF Input Scaling: Set DTMF detection input scaling value.

Dual Phone Connection:

- Press ‘#*#’ to pair first phone.

- After connected, press ‘#*#’ again to pair second phone.

(Note: Maximum pairing time is 300 seconds.)

- After connected, both phones will be automatically re-connected once in range.

(Note:  It will take up to 30 seconds to establish Bluetooth connection)

Hook Flash : Three way talk toggling, answer incoming call during Siri, Call waiting answering.

During streaming state, stop the streaming.

Dialing Plan: (Option to enable 5 seconds timeout or only use # to trigger dialing)

10 digits	: 	first digit is not 0 nor 1.

11 digits	: 	first digit is 1 and second digit is not 1.

3 digits	: 	n11 (n = 2 ~ 9)

>10 digits:	first digit is 0 end with # and length < 20

<10 digits: 	end with # and length >= 3

No digit for 10 seconds will trigger the dialing if stored digit number is > 5,

Rotary feature is 5 seconds.

*67 for no-CID outgoing call.

Call Waiting:

* key or hook flash to answer the new incoming call and hold current call.

* key or hook flash to switch back and forth between calls.

# key to call conference both calls.